## LIMIT SWITCH

##### CLZ-8104S

#### INPUT SPECIFICATIONS

| Operating Speed | 0.5mm-50cm/sec |
| --- | --- |
| Operating Frequency | Mechanical: 120 Operating/minuts Electrical : 30 Operations/minuts |
| Contact Resistance | 25mΩ min. (Initial Value) |
| Insulation Resistance | 100mΩ min. (below 500VDC) |

#### OUTPUT SPECIFICATIONS

| Dielectric Strength | 1000VAC, 50/60Hz for 1 minute between terminals of the same polarity |
| --- | --- |
|  | 1500VAC, 50/60Hz for 1 minute between current-carrying and non-current-carrying metal parts |
|  | 1500VAC, 50/60Hz for1 minute between each terminal and ground |
| Vibration | 10-55Hz,1.5mm double amplitude |  |

#### ENVIRONMENT

| Ambient Temperature | -5~ 65oC (with no icing) |
| --- | --- |
| Humidity | ~95% RH |
| Life | Mechanical : 10 Million Operation above Electrical : 500,000 Operation above |
| Shock | Mechanical durable : 1,000m/sec2 (about 100G'S) Malfunction : 300m/sec2 (about 30G'S) |

#### STANDARDS

| Weight | About 130 to 190g |
| --- | --- |
| EMC Standards | EN60947-5-1 |
| Degree Of Protection | IP65 |

#### Applications

| Industrial Control Panel | Injection Moulding Machines |
| --- | --- |
| Elevator Control Panel | Industrial Machinery |